# BasaltOS Development Roadmap

A Truly Portable Embedded RTOS

Version 1.0 • January 2026

# Vision

BasaltOS aims to be a lightweight, portable embedded operating system that scales from 8-bit microcontrollers to 32-bit+ processors with a consistent API. The goal is to provide enough value-add over bare-metal programming to justify its footprint, while remaining flexible enough to run on everything from an ATmega328P blinking an LED to an STM32 or RP2040 running building automation systems.

## Core Principles

- Platform Agnostic: Write once, run anywhere (with appropriate feature subsets)

- Modular Architecture: Enable/disable features based on platform capabilities

- Hardware Abstraction: Clean HAL layer isolates platform-specific code

- Developer-Friendly: High-level APIs and scripting support where resources allow

- Scalable: From minimal cooperative scheduler to full preemptive multitasking

# System Architecture

BasaltOS uses a layered architecture to maintain portability:

| Layer | Description |
| --- | --- |
| Applications | User apps, shell (bsh), MicroPython/Lua runtimes |
| BasaltOS APIs | High-level APIs for GPIO, timers, filesystem, networking, etc. |
| Kernel | Scheduler, IPC (queues, semaphores), timers, memory management |
| HAL | Hardware abstraction: GPIO, UART, SPI, I2C, ADC, PWM, timers |
| Arch | CPU-specific: context switching, interrupt handling, atomics |
| Hardware | Physical MCU/MPU/CPU (AVR, ARM Cortex-M, Xtensa, RISC-V, etc.) |

# Target Platforms

## Initial Platform: ESP32 (Phase 1)

Starting with ESP32 provides the best foundation:

- 32-bit Xtensa dual-core architecture

- Sufficient RAM/Flash for full feature development

- Built-in WiFi and Bluetooth

- Mature ESP-IDF toolchain and documentation

- Low cost and wide availability

Current Status: v0.x with CLI shell, SPIFFS filesystem, MicroPython runtime, SD card support, and basic app lifecycle.

## Expansion Platforms

| Platform | Phase | Architecture | Rationale |
| --- | --- | --- | --- |
| RP2040 | 2 | ARM Cortex-M0+ dual-core | Validates ARM portability, different from Xtensa, popular platform |
| STM32 | 3 | ARM Cortex-M3/M4/M7 | Industry standard, proves ARM vendor portability, wide range of capabilities |
| ATmega328P | 4 | AVR 8-bit | Ultimate minimalism test, Arduino ecosystem, proves true 8-bit scalability |
| PIC | 4 | 8/16/32-bit variants | Validates 8-bit portability across architectures, industrial/automotive use |
| RISC-V | 5 | RISC-V 32/64-bit | Future-proof, open ISA, growing ecosystem (ESP32-C3, SiFive, etc.) |

# Development Phases

## Phase 1: ESP32 Foundation (Current)

Timeline: Q1-Q2 2026

Goals:

- Stabilize build system and get ESP32 fully operational

- Complete BLE keyboard input support for shell

- Refine MicroPython integration and basalt.* APIs

- Implement modular input subsystem (UART, BLE, touch, etc.)

- Define and document HAL boundaries for portability

- Create UI/graphics API for display output

- Add app permissions and resource limits

- Implement package signing for security

Deliverables:

- BasaltOS v1.0 for ESP32

- Complete HAL specification document

- Developer SDK and documentation

- Example apps demonstrating key features

## Phase 2: ARM Cortex-M Portability

Timeline: Q3-Q4 2026

Target Platforms: RP2040, STM32F4

Goals:

- Implement ARM Cortex-M arch layer (context switch, interrupts)

- Port HAL to RP2040 SDK and STM32 HAL

- Validate kernel portability across different ARM cores

- Create board configuration system for multiple platforms

- Test cross-platform app compatibility

Deliverables:

- BasaltOS v2.0 for RP2040 and STM32

- Refined HAL proving ARM vendor independence

- Automated platform testing framework

## Phase 3: 8-bit Minimalism

Timeline: Q1-Q2 2027

Target Platforms: ATmega328P, PIC18F

Goals:

- Create minimal kernel profile (cooperative scheduler, no filesystem)

- Implement AVR arch layer in assembly

- Optimize memory footprint for 2KB RAM constraints

- Prove value-add over bare metal on resource-constrained devices

- Port to PIC to validate 8-bit cross-architecture portability

Deliverables:

- BasaltOS v3.0 minimal profile

- 8-bit HAL specification

- Configuration guide for feature scaling

## Phase 4: RISC-V and Ecosystem

Timeline: Q3-Q4 2027

Target Platforms: ESP32-C3/C6, SiFive cores

Goals:

- Port to RISC-V architecture

- Expand app ecosystem with community contributions

- Create app marketplace/repository

- Formalize BasaltOS API specification

- Build community and documentation

Deliverables:

- BasaltOS v4.0 for RISC-V

- Public API specification v1.0

- App store and package manager

- Community forum and contribution guidelines

# Core Technical Components

## 1. Kernel

### Scheduler

- Full profile (32-bit): Preemptive priority-based scheduler with round-robin

- Minimal profile (8-bit): Cooperative scheduler with explicit yield points

- Configurable tick rate (1-1000 Hz)

- Stack overflow detection (where resources allow)

### Inter-Process Communication

- Message queues

- Semaphores and mutexes

- Event flags

- Shared memory regions (on platforms with MMU/MPU)

### Memory Management

- Dynamic allocation with configurable heap

- Memory pools for fixed-size allocations

- Stack monitoring and protection

- MPU support where available (STM32, etc.)

## 2. Hardware Abstraction Layer (HAL)

The HAL provides a unified API across all platforms:

| Peripheral | API Functions |
| --- | --- |
| GPIO | init, set_mode, read, write, toggle, set_interrupt |
| UART | init, send, receive, set_baudrate, register_callback |
| SPI | init, transfer, set_clock, set_mode |
| I2C | init, read, write, scan, set_speed |
| Timers | init, start, stop, set_period, register_callback |
| ADC | init, read, set_resolution, set_reference |
| PWM | init, set_duty, set_frequency, start, stop |

## 3. Filesystem Support

- Full profile: SPIFFS, FATFS, LittleFS

- Minimal profile: EEPROM key-value store

- Virtual filesystem layer for unified API

- Support for SD cards, SPI flash, internal flash

## 4. Application Runtime Support

### Native C/C++ Apps

- Direct access to all BasaltOS APIs

- Minimal overhead

- Full hardware control

### Scripting Languages (32-bit only)

- MicroPython: Currently implemented, basalt.* API bindings

- Lua: Planned, lightweight alternative

- WASM: Future consideration for sandboxed apps

## 5. Networking Stack (Optional)

- WiFi support (ESP32, RP2040W)

- Bluetooth/BLE support

- TCP/IP stack (lwIP integration)

- MQTT, HTTP client libraries

- OTA update support

# Why BasaltOS Over Bare Metal?

BasaltOS must provide clear advantages to justify its footprint:

| Feature | Bare Metal | BasaltOS |
| --- | --- | --- |
| Multitasking | Manual state machines | Preemptive scheduler |
| Portability | Rewrite for each platform | Write once, recompile |
| App Updates | Reflash entire firmware | Install/update apps separately |
| Development | C/C++ only, hardware-specific | Python/Lua for rapid prototyping |
| Debugging | Platform-specific tools | Unified shell and debugging |
| Resource Protection | None - bugs crash system | MPU/permissions isolate apps |

# Configuration System

BasaltOS uses a hierarchical configuration system to enable/disable features:

### Core Kernel Options

- CONFIG_BASALT_MAX_TASKS - Maximum concurrent tasks (4-32)

- CONFIG_BASALT_TASK_STACK_SIZE - Default stack per task (256-4096 bytes)

- CONFIG_BASALT_TICK_HZ - Scheduler tick rate (10-1000 Hz)

- CONFIG_BASALT_SCHEDULER_TYPE - Preemptive or cooperative

- CONFIG_BASALT_ENABLE_FPU - Floating point support

### Feature Modules

- CONFIG_BASALT_ENABLE_FILESYSTEM - File system support

- CONFIG_BASALT_ENABLE_NETWORKING - WiFi/BLE networking

- CONFIG_BASALT_ENABLE_MICROPYTHON - MicroPython runtime

- CONFIG_BASALT_ENABLE_LUA - Lua runtime

- CONFIG_BASALT_ENABLE_GRAPHICS - Display/UI support

- CONFIG_BASALT_ENABLE_USB - USB device/host support

### Input Subsystem

- CONFIG_BASALT_INPUT_UART - UART input

- CONFIG_BASALT_INPUT_BLE_KEYBOARD - Bluetooth keyboard

- CONFIG_BASALT_INPUT_USB_KEYBOARD - USB keyboard

- CONFIG_BASALT_INPUT_TOUCH - Touchscreen

- CONFIG_BASALT_INPUT_GPIO_BUTTONS - Hardware buttons

## Example Platform Profiles

| Feature | ATmega328P | RP2040 | ESP32 |
| --- | --- | --- | --- |
| Max Tasks | 4 | 16 | 32 |
| Scheduler | Cooperative | Preemptive | Preemptive |
| Filesystem | EEPROM KV | LittleFS | SPIFFS/FATFS |
| Scripting | None | Lua | MicroPython |
| Networking | None | WiFi (2040W) | WiFi/BLE |

# Success Metrics

## Technical Metrics

- Portability: Support at least 5 different CPU architectures by v4.0

- Footprint: Kernel <8KB on minimal profile, <64KB on full profile

- Performance: Context switch <50 CPU cycles on ARM Cortex-M

- API Stability: No breaking changes to HAL after v1.0

- Test Coverage: >80% code coverage on core kernel

## Community Metrics

- 100+ stars on GitHub by v2.0

- 10+ community-contributed apps by v3.0

- Active forum/Discord with 50+ members by v4.0

- 5+ third-party board ports by v4.0

# Risks and Mitigations

| Risk | Impact | Mitigation |
| --- | --- | --- |
| Feature creep makes OS too heavy | Can't run on resource-constrained devices | Strict configuration system, maintain minimal profile |
| HAL abstractions leak platform details | Portability compromised | Design HAL first, validate with 3+ platforms early |
| Competition from FreeRTOS, Zephyr | Low adoption | Focus on developer experience and simplicity |
| 8-bit port proves too limited | Can't demonstrate true scalability | Accept 16-bit minimum, emphasize 32-bit strength |
| Lack of community adoption | Project stagnates | Early documentation, example projects, engage makers |

# Conclusion

BasaltOS aims to fill the gap between bare-metal programming and full-featured operating systems like Linux. By providing a portable, modular RTOS that scales from 8-bit microcontrollers to 32-bit+ processors, BasaltOS will enable developers to write once and deploy across a wide range of embedded platforms.

The phased approach — starting with ESP32, expanding to ARM platforms, validating on 8-bit systems, and ultimately supporting RISC-V — ensures that the architecture evolves based on real-world constraints and requirements. Each phase builds on the previous one, proving portability and refining the abstractions that make cross-platform development practical.

Success will be measured not just by technical metrics like footprint and performance, but by the strength of the developer community and ecosystem that grows around BasaltOS. By focusing on developer experience, comprehensive documentation, and a clear value proposition over bare-metal development, BasaltOS has the potential to become a go-to platform for embedded systems development.